الوثائق النمطية للتوريدات البسيطة                                                                                                                  القسم السادس: جدول المتطلبات

- المواصفات الفنية

Equipment 4x4: Mobile Workshop

EQUIPMENT: Mobile Workshop

QUANTITY: 1 unit.

| No. | DESCRIPTIONS | TENDERER'S SPECIFICATION |
| --- | --- | --- |
| 1 | MANUFACTURER : To be specified |  |
| 2 | MODEL : To be specified |  |
| 3 | COUNTRY ORIGIN : To be specified |  |
| 4 | SPECIFICATIONS :- |  |
|  | 1-Truck |  |
|  | a- Engine *Type: Water – cooled, 4cycle diesel engine *No. Of cylinders: 6 *Max. Output: up to 200 HP *Lubrication system: Forced by Gear pump *Air Cleaner: Dry type *Electric system: 24V system |  |
|  | . b- Transmission *Front: 5 speed or more *Reveres: 1 speed or more |  |
|  | c- Axle *Front: Full floating or I beam *Rear: Full floating |  |
|  | d- Brake *System: Full air or hydraulic brake *Parking brake: Mechanically operated |  |
|  | e- Clutch *Single dry plate clutch |  |
|  | 2-Workshop |  |
|  | a- Body *Construction: Mainly steel welded construction, Aluminum heat insulated Van with 4 windows *3 door on rear and side |  |
|  | b- Attachment *Monorail crane fixed overhead in the van body *Rear extension with 450kg-chain block |  |
|  | c- Equipment for maintenance 1- A-C Generator/Welder: 1 unit *Type: diesel engine driven (7hp) *Electrical system 220v, 50Hz, 2- Oxy-Acetylene cutting and brazing equipment:1 set *Oxygen cylinder: 1 unit * Acetylene cylinder: 1 unit *Pressure regulator for Oxygen: 1unit *Pressure regulator for Acetylene: 1 unit *Cutting torch set: 1 set *Welding torch set: 1 set *Oxygen hose (10-15m): 1 unit *Acetylene hose (10-15m): 1 unit * Welding glass: 1 unit *Cylinder opener spanner: 1 unit 3- Mechanic tool kit: 2 set *Open wrenches: 6x7~30x32m/m: 18 pcs *Allen wrenches: 5/32~5/8: 9 pcs *Box wrenches: 10x12~29x32m/m: 7 pcs * Box wrenches: 10x12~29x32m/m: 7 pcs {small size} *Adjustable wrenches: 100, 200, 300, 450 m/m: 4 pcs *Socket wrenches: 0.5 sq. drive 9m/m~26m/m: 13 pcs *Combination wrench: 10~32m/m: 14 pcs *Straight drives: 1 set *Philip drives: 1 set *Other tools:20 pcs 4- Lubrication equipment:1 set *Oil bucket pump: 1unit *Grease bucket pump: 1unit *Grease gun: 1 unit *Oil syringe: 1unit *Drain plug wrench: 1pc 5- Diesel engine service equipment: 1 set *Nozzle tester: 1 unit *Compression gauge: 1 unit *Adjustable remer: 1 unit *Screw extractor set: 1 set *Cylinder gauge: 1 unit *Thickness gauge: 1 unit *Dial gauge: 1 unit 6- Electric tool kit: 1 set *Soldering iron {200W}: 1 unit * Soldering paste: 1 set * Soldering {500g}: 1 set *Quick method battery charger: *AC/DC Volt-ampere tester: 1 unit *Hand tachometer: 1 unit *Circuit tester: 1 unit *Varnish caliper: 1 set *Micrometer: 1 set. 7- Light: 1 set *Floor light: 1 unit *Flash light: 1 unit 8- Jack (20 ton): 2 unit 9- other common mechanical tool set: 2 set 10- Others *Air compressor with motor {2~2.5kw}: 1unit *Manual hydraulic press {20 ton}: 1 unit *Workbench with fixed electric grinder, electric drill & clamp: 1 set *Washing bench: 1 unit *Manual fuel pump: 1 unit *Aluminum ladder: 1 unit *Tube punctual tool kit with iron: 1 set *Electric drill {portable}: 2 unit * Electric hand grinder: 2 set *Gouges {small, medium &large}: 1set *Pliers: 1 set *Mechanic headset: 1 unit *Monkey wrench: 1 set *Oiler 0.25liter: 1 unit *Torque wrench: 1set *Puller {small, medium &large size} with 2 arm: 1 set * Puller {small, medium &large size} with 3 arm: 1 set *Bearing/Gear puller {small, medium &large size}: 1set *Piston ring remover: 1 set * Piston ring assembly: 1 set *Engine liner puller: 1 set *Valve bush remover: 1 set *Bolt thread tools: 1 set *Hole thread tools: 1 set *Portable grinder {inner use}: 1 unit *Fire extinguisher: 1 unit |  |

### يجب تعبئة المواصفات في الجدول من قبل المشاركين في المناقصة كما وردت أعلاه وأي بيانات أخرى تتعلق بالمواصفات يمكن إرفاقها